BlueLine One

User Guide

ESP32-S3 Round AMOLED OBD-II Gauge

Safety: BlueLine One is an experimental DIY automotive display. Do not interact with the display while driving. Always verify critical vehicle information with proper diagnostic equipment.

# 1. Overview

BlueLine One is a custom round AMOLED automotive gauge built around the Waveshare ESP32-S3 1.75-inch display. It connects to a compatible BLE OBD-II adapter and displays live vehicle data.

Current gauge screens include:

4-in-1 / All-In-One gauge

RPM

Speed

Throttle

Intake Air Temperature

Coolant Temperature

Battery Voltage

TPMS interface

Airflow / MAF

MPG

Chronograph

## What the 4-in-1 Screen Shows

RPM shift-light arc

Vehicle speed

Throttle position

Intake air temperature

# 2. Getting Started

## Before Powering On

Install the BLE OBD-II adapter in the vehicle's OBD-II port.

Turn the vehicle ignition on.

Connect BlueLine One to a stable USB-C power source.

Wait for the boot sequence to complete.

Note: A poor-quality 12-volt USB adapter or electrical noise can affect touch performance. For troubleshooting, test the display with a USB power bank.

## Connecting to the OBD-II Adapter

Long-press the Bluetooth icon for approximately 3 seconds.

Select Scan.

Choose the BLE OBD-II adapter from the results.

Wait while BlueLine One connects and initializes the adapter.

Return to the gauge screens after connection is complete.

The selected adapter can be saved for later use. The Bluetooth icon appears blue when connected and gray when disconnected.

# 3. Touch and Button Controls

| Action | Function |
| --- | --- |
| Swipe left or right | Move between gauge screens |
| Swipe up or down | Change the ring-light color for the current gauge |
| Double-tap | Cycle display brightness |
| Long-press Bluetooth icon | Open the BLE menu |
| Long-press speed value/unit | Toggle MPH and KPH |
| Long-press temperature value | Toggle Fahrenheit and Celsius |
| Long-press TPMS unit | Toggle PSI and kPa |
| Long-press RPM gear icon | Open shift-light settings |
| Long-press MPG area | Reset estimated average MPG |

## Hardware Buttons

| Button | Chronograph Function |
| --- | --- |
| PWR | Start or stop the chronograph |
| BOOT | Reset the chronograph |
| Screen tap while on Chronograph | Reset the chronograph |

# 4. Gauge Navigation Order

The current swipe order is:

1. 4-in-1 / All-In-One

2. RPM

3. Speed

4. Throttle

5. Intake Air

6. Coolant

7. Battery

8. TPMS

9. Airflow

10. MPG

11. Chronograph

12. Back to 4-in-1 / All-In-One

# 5. Gauge Screen Guide

## 4-in-1 / All-In-One

Combines the RPM shift-light arc, speed, throttle position, and intake air temperature on one screen. Long-press the speed area to change MPH/KPH, the intake-air area to change °F/°C, or the Bluetooth icon to open the BLE menu.

## RPM

Displays engine RPM and the programmable shift-light arc. Long-press the gear icon to open shift-light settings.

## Speed

Displays vehicle speed. Long-press the speed value or unit to switch between MPH and KPH.

## Throttle

Displays throttle position as a percentage and segmented bar.

## Intake Air

Displays intake air temperature. Long-press the value to switch between Fahrenheit and Celsius.

## Coolant

Displays engine coolant temperature. Long-press the value to switch between Fahrenheit and Celsius.

## Battery

Displays adapter or control-module voltage. The icon fill and voltage color change according to the measured voltage.

## TPMS

Displays the four tire-pressure positions and supports PSI/kPa switching. Live Mazda TPMS support is experimental.

## Airflow

Displays mass airflow in grams per second using OBD-II PID 0110.

## MPG

Displays instant MPG and an estimated average MPG. Long-press the MPG area to reset the average.

## Chronograph

Provides a stopwatch-style chronograph with a tachymetre-inspired face. Use the PWR button to start/stop and the BOOT button or screen tap to reset.

# 6. Ring-Light Colors

Swipe up or down on a gauge screen to change its ring-light color. Each gauge stores its own selection.

Cyan

Blue

Red

Yellow

Green

Purple

Magenta

Synthwave

No Color / Ring Off

# 7. Adjusting the RPM Shift Lights

Open the RPM gauge.

Long-press the gear icon for approximately 3 seconds.

Use NEXT and PREV to select a shift-light step.

Tap MORE or LESS to adjust by 25 RPM.

Hold MORE or LESS to adjust repeatedly in 250-RPM increments.

Select SAVE to store the settings.

Select BACK to return to the RPM gauge.

Note: The 4-in-1 screen uses the same saved RPM shift-light settings as the individual RPM gauge.

# 8. Display Brightness

Double-tap the display to cycle through:

Dim

Medium

Bright

# 9. BLE Menu

The BLE menu is opened by long-pressing the Bluetooth icon.

Scan: Search for nearby BLE devices.

Reconnect: Attempt to reconnect to the saved adapter.

Forget: Remove the saved adapter.

Back: Return to the previous gauge.

# 10. MPG Information

Instant MPG is estimated from vehicle speed and mass airflow data. Estimated average MPG is calculated from accumulated trip distance and estimated fuel use.

Note: MPG values are estimates and are not a replacement for the vehicle's calibrated fuel-economy display.

# 11. Troubleshooting

## Bluetooth icon stays gray

Confirm the adapter is powered, the ignition is on, and the adapter supports BLE. Open the BLE menu and scan again.

## Gauge values remain as dashes

Verify the BLE connection and allow time for OBD-II initialization. Some PIDs may not be supported by every vehicle.

## Touch is unreliable in the vehicle

Test with a USB power bank. Try a better-quality automotive USB adapter, shorter cable, or improved grounding.

## Display uploads but remains blank

Verify the correct board settings, display pins, Arduino GFX library version, and CO5300 display configuration.

## PWR button does not operate the chronograph

Confirm XPowersLib is installed and the sketch includes XPowersLib with BLUELINE_HAS_XPOWERS set to 1.

## TPMS shows no pressure

TPMS support is experimental and may require vehicle-specific extended PIDs and CAN headers.

# 12. Safety and Limitations

Do not operate or configure the display while driving.

Mount the display so it does not block the driver's view or interfere with airbags.

Do not rely on BlueLine One for safety-critical warnings.

Vehicle data may be delayed, unavailable, or inaccurate.

BLE adapter compatibility varies.

TPMS and manufacturer-specific functions remain experimental.

# 13. Quick Reference

| Need to... | Do this |
| --- | --- |
| Change gauges | Swipe left or right |
| Change ring color | Swipe up or down |
| Turn ring off | Swipe to No Color / Ring Off |
| Change brightness | Double-tap |
| Connect OBD-II adapter | Long-press Bluetooth icon, then Scan |
| Change MPH/KPH | Long-press speed value or unit |
| Change °F/°C | Long-press the temperature value |
| Change PSI/kPa | Long-press the TPMS unit |
| Adjust shift lights | Long-press the gear icon on the RPM screen |
| Reset average MPG | Long-press the MPG area |
| Start/stop chronograph | Press PWR |
| Reset chronograph | Press BOOT or tap the Chronograph screen |

BlueLine One • Edge 3D Solutions